# Torque Calculation for 6 kg Robot on Tiles

Estimating the required torque for robot’s displacement to be close to human pace while tracking, we used:

• Mass m = 6 kg

• Ground: flat terrain

• Number of driven wheels = 2

• Wheel radius r = 0.065 m (130 mm diameter)

• Chosen linear acceleration a ≈ 0.3 m/s²

1. Acceleration force:

F_acc = m · a = 6 × 0.3 = 1.8 N

2. Rolling resistance on tiles (C_rr ≈ 0.02):

F_rr = C_rr · m · g = 0.02 × 6 × 9.81 ≈ 1.18 N

3. Total drive force:

F_total ≈ F_acc + F_rr ≈ 1.8 + 1.18 ≈ 3.0 N

4. Torque at the wheels:

T_total = F_total · r = 3.0 × 0.065 ≈ 0.195 N·m

5. Torque per motor (with 2 driven wheels):

T_per_motor ≈ T_total / 2 ≈ 0.195 / 2 ≈ 0.0975 N·m ≈ 0.1 N·m

Therefore, each motor should be able to provide about 0.1 N·m of continuous torque. For this, we selected the motors with a safety factor of around 2, this value as stall torque to ensure a safe margin.

| PWM duty (d) | Approx. torque (T(d)) |
| --- | --- |
| 30% | 0.059 N·m |
| 40% | 0.078 N·m |
| 50% | 0.098 N·m (just below required) |
| 60% | 0.118 N·m |
| 70% | 0.137 N·m |
| 80% | 0.157 N·m |
| 90% | 0.176 N·m |
| 100% | 0.196 N·m |

We need around 0.1N.m